|  | Rad Hesgar Melal Co. ELECTRICAL CONDUCTIVITY (EC) SENSOR AND TRANSMITTER ONLINE ELECTRICAL CONDUCTIVITY METER |
| --- | --- |

| FEATURES - Compact design with internal transmitter - Online & real-time measurement - High accuracy - Electromagnetic isolation - Strong resistance to corrosion - Polarization resistance - Digital/Analog output - 316L stainless steel electrode | APPLICATIONS - Environmental protection - Aquaculture - Water source monitoring - Environmental monitoring - Sewage treatment |
| --- | --- |

TECHNICAL SPECIFICATION

| Description | Explanations |
| --- | --- |
| Measuring medium | Liquid |
| Principle | Frequency conversion method |
| Temperature compensation | Automatic |
| Range | 0 ~ 20 mS/cm (any range of 0-300 mS/cm can be customized, 1ms/cm = 1000µS/cm) |
| Supply | 12 ~ 30 VDC (power consumption<0.2W) |
| Accuracy | ±3% |
| Response time | 1s |
| Output Signal | Modbus RS485 4-20 mA (Depends on customer order) |
| Electrode | 316L stainless steel |
| Operating temperature | -5 ~ +65 (̊C) |
| Probe material | 316L and ABS |
| Ingress protection | Probe: IP68 Body: IP65 |
| Cable length | Default:2m |

WIRING DIAGRAM for Modbus-RS485 output

WIRING DIAGRAM for 4-20 mA output

DEMENSIONS (in mm)